# BFD-CNN: An Efficient Ubiquitous CNN with Tunable Input-Size for Bearing Fault Diagnosis

Jungan Chen1, Jean Jiang2, Xinnian Guo3, Lizhe Tan4

Abstract—Deep learning can automatically learn complex features of input data and is an effective way for bearings fault diagnosis. Because CNN is successful used in image classification, many existing researches use image of vibration signal or time-frequency information obtained through STFT, Wavelet Transform and EMD, etc show promising results. However, their complex CNN structure is not ubiquitous and efficient enough with different dataset. Furthermore processing with Wavelet Transform and EMD is less efficient than STFT. In this work, we analyze the low bound of 2D data input size by considering the relation between the characteristic vibration frequencies and window size of STFT to guide choosing of input size. Then a general CNN structure is designed. Through experiments applied for four dataset, the proposed method is ubiquitous and not sensitive with parameters by considering the low bound of input size. Surprisingly, all the prediction accuracies of four dataset achieved 100% in 10 times running without redesigning the CNN structure.

Index Terms—bearing fault diagnosis, deep learning, CNN, STFT.

# INTRODUCTION

C

ondition based maintenance(CBM) also referred to as Prognostics and Health Management (PHM) is a maintenance strategy which monitors the health condition of machinery in real time and makes an optimal maintenance decision based on condition monitoring information. Henriquez et al.[1] point that CBM usually includes four stages data acquisition, signal processing, decision support system and fault diagnosis/prediction. Rai et al.[2] introduce that CBM include three main components which are condition monitoring,  health prognosis, fault diagnosis . Condition monitoring is a basic component. It will collect relevant information about the machine condition which is vibration signals, motor current signals, acoustic emission signals, and more recently, stray flux[3]. Health prognostics aim to predict

The date submitted for review: 04/28/2019. This work was supported in part by Zhejiang Provincial Natural Science Foundation (LY16G020012) and Zhejiang Province Public Welfare Technology Application Research(LGF19F010002).

Jungan Chen is with the Department of Electronic and Computer Science, Zhejiang Wanli University, Ningbo 310023 China. (e-mail: friendcen21@hotmail.com).

Jean Jiang is with the College of Technology, Purdue University Northwest, Indiana 46323 USA. (e-mail: jjiang@pnw.edu).

Xinnian Guo is with Department of Electronic Information Engineering,Huaiyin Institute of Technology, Huaian 223003, China. (e-mail: xnguo@hyit.edu.cn).

Lizhe Tan, Xuen Peng, and Jintao Hou are with the Department of Electrical and Computer Engineering, Purdue University Northwest, Indiana 46323 USA. (e-mail: lizhetan@pnw.edu).

.

the remaining useful life (RUL) of machinery based on the historical and on-going degradation trends observed from condition monitoring information[4]. And fault diagnostics is the process of typifying the status of damage through detection, isolation and identification using the data collected from the continuous health monitoring of the rotating machinery. Diagnosis can be regarded as a posterior event analysis[2].

In industry factory, rolling element bearings(REB) are the most common machine elements in almost all kinds of rotating machinery and health conditions of REB have considerable impacts on machines. According to a literature review, 45–55% of broken machines are caused by bearing faults. Thus, condition monitoring and fault diagnosis of bearings are significant tasks in the industry[2] [5].

In general, the fault diagnosis methods can be classified into model-based, signal-based, knowledge-based, and hybrid/active methods [6]. The knowledge-based methods, which are also named as data-driven methods, require a large volume of historic data to establish the fault modes about the systems without priori known models or signal patterns [7].

In conventional data-driven method, it usually consists of the three stages: handcrafted feature design, feature extraction/selection and model training.

Normally, handcrafted feature design is usually based on signal process method such as Fourier transform (FT), short-time Fourier transform(STFT), wavelet transform(WT), wavelet package transform(WPT) and Hilbert-Huang transform(HHT), Empirical Mode Decomposition (EMD)[8][9] [10]. After the right set of features are designed, these features are fed into some shallow machine learning algorithms such as EMD+SVM[10], HHT+SVM[11], DWT+KNN [12], WPT+KNN[13], WT +Naive Bayes (NB)[14].

Except signal process based methods, many existing works have been done with the image processing method. Chong converted 1-D vibration signals into 2-D gray-level images and significant features from vibration signals are extracted through the scale invariant feature transform (SIFT) algorithm to generate the faulty symptoms[15]. Kang and Kim proposed a 2-D gray-level images representation based on Shannon wavelets and extracted image textures by generating global neighborhood structure. Then multiclass SVMs is used for identifying faults in the induction machine[16]. Lu et al. proposed a conversion from signals to images using bi-spectrum technology and t-SNE is to reduce the dimension of Features extracted with SURF. Then a probabilistic neural network is used to classify the images[17].

However, manually designing features for a complex domain requires a great deal of human labor and domain knowledge[18]. Nowadays, deep learning can automatically learn complex features of input data, so it has emerged as an effective way to overcome the above drawback.

LeCun et al. first designed a Convolution neuron network(CNN) and optimized the model using an error-gradient algorithm. Due to their unique ability to maintain initial information regardless of shift, scale and distortion invariance, CNNs are widely used in image classification [19, 20].

There are two type CNN applied in fault detection. One is 1-D CNN, another is 2-D CNN.Ince and Eren et al. [21] [22] applied1-D CNN to real-time motor fault diagnosis.  Abdeljaber et al. used a 1-D CNN for real-time structural damage detection [23].

As for 2-D CNN, Guo et al. propose a hierarchical learning rate adaptive deep CNN (ADCNN) to accomplish fault diagnosis and severity. The first layer is based on the classical LeNet5 models [24]

Jia et al. [25] use Stack Auto Encoder(SAE) to learn the useful information from  obtained frequency spectra of rotating machinery. Liu et al. [26] employ stacked sparse auto encoder to automatically extract the fault features of sound spectrograms using STFT, and softmax regression is adopted as the method for classifying the fault modes. Wang et al. [27] use wavelet scalogram images as an input into CNN to detect faults within a set of vibration data. Similarly, three time-frequency analysis methods STFT, WT, and HHT are used to generate image representations of the raw signal to feed into CNN [9]. It use three repeat CCP-layers, each CCP-layer consists two consecutive convolution layers without a pooling layer between them.

Vibration signal is normalized and transformed to vibration image. Wen et al. [7] feed these vibration images into CNN, which consist four successive convolution and pooling layer. Similarly, Hoang et al. [5] use a CNN structure consisting two successive convolution layers and pooling layers.

As reviewed above references, both conventional and deep learning methods have used image as input data, so it is a general idea to feed 2D or image data into CNN for fault diagnosis. Although many related works have achieved successful results, there are still many issues to be handled. Firstly, Refs [5,7] directly use vibration signal as image data and cannot capture frequency information which is important to non-stationary signal. Secondly, although STFT, WT, HHT is used to generate spectrum or scalogram image, these color images scaled by using bilinear interpolation[9, 27] as input of CNN will cause efficiency problem. Lastly, the design of network structure is still an issue. The CNN structure of Refs[7,9] are too complicated because of too many convolution layers. And different CNN structure or parameters are required to be redesigned according to the different data set.

As we know STFT is a simple, easy to apply signal transformation method that can transform time domain signals into time-frequency space. And it is efficient than WT,HHT, EMD. In this work, we will use STFT with rectangle and no overlap window to achieve power spectrum. Then a efficient and general CNN for Bearing Fault Diagnosis (BFD-CNN) is proposed, which is suit for many different data set without any change of CNN structure.

The main contribution of this work is as follows:

1) By using STFT, power spectrum with normalization is used as input of CNN. As we know STFT cannot adapt the widow size and will lose much information such as short pulse or low frequencies, a tunable parameter W is proposed to solve the problem with the guide of theoretic analyze, which consider the relation between the characteristic vibration frequencies and window size of STFT

2) A ubiquitous CNN is proposed and successful to apply in four dataset with the same CNN structure and can achieve 100% prediction accuracy in ten times running.

3) The proposed CNN method for Bearing Fault Diagnosis, called BFD-CNN, is less sensitive with the parameter W by considering the low bound W.

The rest of this paper is organized as follows: Section II describes the detail of our method including the framework of BFD-CNN, the design principle of STFT with low bound W through the theoretic analysis and the model of CNN. In section III, comparison experiments with four dataset are conducted and the results are presented and analyzed. Finally, Section IV makes a conclusion and suggests topics for future research.

# Our proposed method

In this work, a general CNN based on SFFT (BFD-CNN) is proposed for Bearing Fault Diagnosis in Fig.1. The raw signal is randomly segmented to many signal frames with W*W points, which can take as a function of data augmentation to increase the number and diversity of samples. Of course, W*W could be replaced with W*H. But for simplify, we just use one parameter W=H.

Fig. 1.  Framework of BFD-CNN.

After segmentation, every signal frame is applied with SFFT. The signal frame will be split into W piece of data block and each data block is applied with FFT. When it is finished, power spectrum with time-frequency information is obtained.

At last, the power spectrum matrix is normalized and fed into a general CNN to training or recognize. The CNN structure can be applied four dataset without any change. It should be noted that not the spectrum image but coefficients matrix is the real data fed into CNN in our proposed method.

## The Principle of Random Segmentation and STFT

When a raw signal is input, traditional STFT will split the signal into many frames with fix window size and overlap size as shown in Fig.2(a). While our proposed will first randomly extracted a segmentation of signal to apply STFT with fix widow size and no overlap size in Fig.2(b). By comparing (a) and (b), the interval betwen two segmentations is different. Traditional STFT is fixed, while our method is non-fixed. It can be taken as a function of data augmentation used in deep learning to increase the number of data and obtain more diversity of spectrum information.

(a) Segmentation with fix window size and overlap size in SFFT

(b) Random segamentation with fix window size before SFFT

Fig. 2.  Different segmentation methods

According to Fourier series analysis, the coefficients of the Fourier series expansion of the peridodic signal x(t) in a complex form are

ck=1T0T0x(t)e-jω0tdt, -∞<k<∞      (1)

Where k is the number of harmonics corresponding to the harmonic frequency of kf0 and ω0=2πT0. To apply equation (1), we substituteT0=NT, ω0=2πT0 and approximate the integration over one period using a summation by substituting dt=T and t=nT. We obtain

ck=1Nn=0N-1x(n)e-j2πknN, -∞<k<∞      (2)

This transform is applied to stationary signals whose properties do not evolve over time. When the signal is nonstationary, the signal is multiplied by a window and truncated into short data frames. By analyzing these short data frames, the output of successive STFTs can provide a time-frequency representation of the signal[28].

For rectangular window of size M, the window’s function is equation (3).

wm=1,0≤m≤M-10,others            (3)

N=M*M                    (4)

cm,k=1Mn=0N-1xnw(m-n)e-j2πknM, k=0,1,…,M  (5)

pm,k=cm,k2,m∈0,M,k∈[0,M]       (6)

pm,k=pm,k-min⁡(p)max⁡(p)-min⁡(p)                (7)

Suppose the signal has N number of data points, the rectangular window and no overlap size are used to improve the efficiency in equation (5). The equation (7) is the normalization of power spectrum for the input of CNN.

As we know STFT cannot adapt the widow size and will lose much information such as short pulse or low frequencies. Therefore small W is always chosen to keep the information of short pulse. But the number of sample point in one segmentation W*W should be large enough to keep the information of low frequencies. For example, in Fig.3, W*W should be great than the number of data Tp between two peaks.

Fig. 3. Low frequency information in vibration signal of bearing

The frequency of these peaks can be analyzed according the characteristic vibration frequencies, which are calculated by the following equations [8, 22]. Outer race fault frequency, inner race fault frequency, ball fault frequency, cage fault frequency are given by equation (8-10) separately. Nball is the number of ball in one bearing. Dball and Dpitch are the diameter of ball and pitch. θ is the contact angle. fr is the shaft speed. fs is the sample frequency. The low bound of W can be obtained by equation (13). The sample factor α should be satisfied with  α>2 according Shannon sampling theorem.

fOR=Nball2∙fr∙(1-DballDpitch∙cosθ)        (8)

fIR=Nball2∙fr∙(1+DballDpitch∙cosθ)        (9)

fball=Dpitch2Dball∙fr∙(1-DballDpitch2∙cos2θ)     (10)

fcage=12∙fr∙(1-DballDpitch∙cosθ)         (11)

Tw=W2/fs>α*1/min⁡([fOR,fIR,fball,fcage])   (12)

W>α*fs/minfOR,fIR,fball,fcage      (13)

## The General CNN Model

The proposed CNN structure is given in Fig.4 and many detail information are given in Table.I. Three successive convolution and pooling layers are used to extract high level features. ReLU is taken as activation function and max pooling layer is for preserving the maximum coefficients because these coefficients contain the main information of fault features.

Fig. 4.  The proposed CNN structure(suppose W=64)

Let the kernel size is F, padding size is P, the stride step is S, the output size is

Wo=W-F+2PS+1              (14)

Suppose the input data matrix is W*W and W=64. The output size of the first convolution layer is (64-3+2*0)/1+1=63.

The loss function uses the categorical cross entropy in equation (15). N is the number of data samples. M is the number classes. yi,c is a binary value and the c’th element of one-hot encoded label of the i’th data sample. pi,c the prediction probability of the c’th element of CNN output since the output layer use softmax[29].

-1Ni=1Nc=1Myi,c*log⁡( pi,c)           (15)

c=1Myi,c=1,yi,c∈{0,1}           (16)

c=1M pi,c=1               (17)

The optimizer is the Adam method because its hyper-parameters have intuitive interpretations and typically require little tuning[30].

TABLE I

Detail information of CNN

| Layer name | CNN Models(Kernel Filter,Activation Function,Strides,Padding) |
| --- | --- |
| C1 | Conv(32*3*3,ReLU,1,None) |
| P2 | Maxpool(2,2,1) |
| C2 | Conv(64*3*3,ReLU,1, None) |
| P2 | Maxpool(2,2,1) |
| C3 | Conv(128*3*3,ReLU,1, None) |
| P3 | Maxpool(2,2,1) |
| F4 | Dense(160,ReLU) |
| F5 | Dense(80,ReLU) |
| Output | Dense(Clsass_Num,Softmax) |

# experiment

In this section, the proposed fault diagnosis method is conducted on four fault diagnosis datasets, which are the famous Case Western Reserve University’s(CWRU) bearing dataset [31], the famous Machinery Failure Prevention Technology (MFPT) society bearing fault dataset[32], the self-priming centrifugal pump dataset and the axial piston hydraulic pump dataset[17]. In every experiment, let the default batch size is 100, number of epochs is 50. All these results are obtained by running 10 times with one GPU. Let Pt is the proportion of the training data in the whole data set. The default value of Pt is 0.8. All the prediction accuracies is the result with test dataset.

## CWRU bearing dataset

As a standard public dataset, its experiments were conducted using two-horsepower reliance electric motor for the acquisition of accelerometer data on both the drive end and fan end bearings, as shown in Figure 4.

Fig. 5.  Test bed for CWRU data set

TABLE II

Classes in cwru data set

| Fault depth | Fault Name | Class Name Used |
| --- | --- | --- |
| 0 | Normal | Normal |
| 0.028 | 0.028-Ball | 28-Ball |
|  | 0.028-InnerRace | 28-IR |
| 0.021 | 0.021-Ball | 21-Ball |
|  | 0.021-InnerRace | 21-IR |
|  | 0.021-OuterRace12 | 21-OR |
|  | 0.021-OuterRace6 |  |
|  | 0.021-OuterRace3 |  |
| 0.014 | 0.014-Ball | 14-Ball |
|  | 0.014-InnerRace | 14-IR |
|  | 0.014-OuterRace6 | 14-OR |
| 0.007 | 0.007-Ball | 07-Ball |
|  | 0.007-InnerRace | 07-IR |
|  | 0.007-OuterRace3 | 07-OR |
|  | 0.007-OuterRace6 |  |
|  | 0.007-OuterRace12 |  |

The bearings support the motor shaft. Single point artificial faults were seeded in the bearing’s inner raceway (IR), outer raceway (OR), and rolling element (ball) (BF) with an electro discharge machining (EDM) operation. Accelerometers sampling at 12 kHz were attached at the 12 o’clock position to the drive end (DE) and fan end (FE) of the motor housing to measure the vibration.

Vibration data was recorded with various engine loads (0 to 3 hp) running at 1730,1750,1772,1797 RPM, motor shaft bearings with faults ranging in depth (none, 0.007, 0.014, 0.021, 0.028 inches), and fault orientation (inner race, rolling element, and outer race). All these data has one normal class and 15 fault classes without considering the different RPM shown in Table.II. But in this study, many classes (fonts with bold, italic ) are merged as the value in the last column of Table.II. Therefore 12 class data are ready to be classified. In this experiment, 10000 data samples are randomly extracted from raw data file with the same probability.

### Signal Vibration Image and SFFT Image

Fig.6 gives the signal, vibration images used by[5,7] and spectrum images used by our method. Most of them are easier to be classified by human eyes than vibration images transformed from raw signal data. But these two group images14-IR~21-OR and 07-OR/21-Ball/21-IR require to be carefully observed.

07-Ball                                  07-IR                                  07-OR

14-Ball                                  14-IR                                  14-OR

21-Ball                                   21-IR                                  21-OR

28-Ball                                  28-IR                                  Normal

Fig. 6.  Different images of CWRU data set

W=32                                   W=64                                  W=96

Fig. 7.  Frequency information with FFT for CWRU data set(21-OR)

### Comparison Results

In this experiment, BFD-CNN is to compare with shallow machine learning and other deep learning methods from references which are many different CNN based methods(CNN-VibrationImage[7],adaptive deep convolution neural network (ADCNN) [24], CNN- STFT[9],CNN-Wavelet[9], 1D-2 LAYER CNN[33]), deep recurrent neural network(DRNN)[ 34], sparse filter [35], a optimization deep belief network with PSO(DBN-PSO)  [36] DBN-based HDN[37], SVM[10]. It can be seen that our method BFD-CNN(W=96) get 100% accuracy in ten running times. Furthermore, the class number to be classified is 12 which is more than other works except DRNN(96.53% accuracy). And CNN-VibrationImage(10 classes) is 99.79, Sparse filter(10 classes) is 99.66, CNN-Wavelet(4 classes) is 99.8. So our method is more efficiency than many state-art methods. The main reason is that vibration image in Fig.6 is more difficult to be distinguished than spectrum image used by our method. As for other similar CNN, their CNN structure is one factor to lead less efficiency.

TABLE III

Comparison with CWRU dataset (Pt=0.8,%)

| Algorithm | Average Acurracy | Class Number |
| --- | --- | --- |
| BFD-CNN(W=96) | 100 | 12 |
| BFD-CNN(W=64) | 99.89 | 12 |
| DRNN [34] | 96.53 | 12 |
| CNN-VibrationImage[7] | 99.79 | 10 |
| ADCNN[24] | 98.1 | 10 |
| Sparse filter[35] | 99.66 | 10 |
| CNN- Wavelet 96×96 [9] with CNN[27] | 99.8 | 4 |
| CNN-STFT( 96×96 [9] | 99.5 | 4 |
| 1D-2 LAYER CNN[33] | 99.75 | 4 |
| DBN-PSO [36] | 87.45 |  |
| DBN Based HDN [37] | 99.03 | 4 |
| SVM[10] | 87.45 |  |

### Sensitive with Samples and W

W number of signal frames are extracted from raw file with different W=32,64,96 and each frame is applied with FFT separately. The results are shown in Fig.7.

According equation (13) and characteristic vibration frequencies given in [38], fs=12000, α=2,rpm=1700,

W>2*fsminfOR,fIR,fball,fcage=2*12000min5.415 ,3.585,0.3983,2.357*170060=46.12

Therefore when w=32<46.12 in Fig.7, the signal obtained can contain less frequency information and the amplitude in spectrum is the lowest.

To further evaluate the effect of different parameters, many experiments with different W=32,64,96 and Pt=0.1,0.5,0.8 are conducted. The results are given in Table.IV.

It can be concluded that the prediction accuracy is proportional with W and Pt. The reason is that more training samples are beneficial to CNN and large W can capture more time-frequency information. Furthermore, the results are not sensitive with W and Pt excepte the situation of 100% accuracy.

In Table.IV, when W=96 and Pt=0.8, 100% accuracy is achieved with Std=0. Furthermore, with the increasing of W, lower Pt can get higher accuracy than Lower W and higher Pt. For example, 99.99%(W=95 and Pt=0.5) is higher than 99.89%(W=64 and Pt=0.8).

In Fig.8, the accuracy iterative curve with different W and Pt are given. The larger W or Pt is, the more quick the converge speed is.

TABLE IV

Prediction Accuracies with different parameters(%)

| W | Pt | Min | Max | Mean | Std |
| --- | --- | --- | --- | --- | --- |
| 96 | 0.8 | 100.00 | 100.00 | 100.00 | 0.00 |
| 96 | 0.5 | 99.96 | 100.00 | 99.99 | 0.01 |
| 96 | 0.1 | 98.34 | 99.50 | 98.97 | 0.35 |
| 64 | 0.8 | 99.60 | 100.00 | 99.89 | 0.15 |
| 64 | 0.5 | 99.66 | 99.98 | 99.84 | 0.10 |
| 64 | 0.1 | 98.42 | 99.44 | 98.99 | 0.29 |
| 32 | 0.8 | 99.60 | 99.85 | 99.69 | 0.07 |
| 32 | 0.5 | 99.16 | 99.56 | 99.39 | 0.14 |
| 32 | 0.1 | 94.22 | 96.54 | 95.25 | 0.76 |

Fig. 8.  The effect of converge speed with different parameters

## MFPT bearing dataset

This famous MFPT dataset comprises data from a bearing test rig (nominal bearing data, an outer race fault at various loads, and inner race fault at various loads) made up of four sets of bearing vibration data: 1) a baseline set with 270 lbs of load, sampled at 97656 Hz for 6 seconds; 2) Three outer race faults(ThreeOR) included 270 lbs of load an outer race faults set with 270 lbs of load, sampled at 97656 Hz for 6 seconds;3) Seven additional outer race(SevenOR) faults were assessed at varying loads: 25, 50, 100, 150, 200, 250, and 300lbs of load, sampled at 48828 Hz for 3 seconds in each file; and 4) Seven inner race faults(SevenIR) were analyzed with varying loads of 0, 50, 100, 150, 200, 250, and 300 lbs of load, sampled at 48828 Hz for 3 seconds in each file. There are additional data files included in the MFPT data set, but they are not used in the experiments. 10000 data samples are randomly extracted from raw data file with the same probability.

### Signal, Vibration Image and SFFT Image

Fig.9 gives the signal, time-frequency information with STFT, vibration image used by[5,7] and spectrum image used by our method. It seems that time-frequency information or spectrum is easier to be distinguished by human eyes than vibration image transformed from raw signal data.

Baseline                                                      SevenIR

SevenOR                                                      ThreeOR

Fig. 9.  Signal and images of MFPT data set

### Comparison Results

BFD-CNN is compared with existing results from referenced works. From Table.V, it can be seen 99.9% is the best results in other research works. But we get 100% accuracy within ten running times. Especially, CNN using the time-frequency information of wavelet[9] does not perform well because of its CNN structure.

TABLE V

Comparison with MFPT dataset (Pt=0.8,%)

| Algorithm | Average Acurracy | Class Number |
| --- | --- | --- |
| BFD-CNN(W=96) | 100 | 4 |
| BFD-CNN(W=64) | 99.99 | 4 |
| CNN-Wavelet 96 × 96 [9] with CNN [27] | 99.9 | 3 |
| CNN- Wavelet 96 × 96 [9] | 99.9 | 3 |
| 1D-3 LAYER CNN[33] | 98.32 | 3 |

### Sensitive with Samples and W

As [32] given, Dball=0.235, Dpitch=1.245, θ=0, Nball=8, fr=25, fs=97656/6 andα=2. The characteristic vibration frequencies can be calculated and the low bound W is 56.66.

fOR=Nball2∙fr∙1-DballDpitch∙cosθ

=82∙25∙1-0.2351.245∙cos0=81.124

fIR=Nball2∙fr∙1+DballDpitch∙cosθ

=82∙25∙1+0.2351.245∙cos0=118.88

fball=Dpitch2Dball∙fr∙1-DballDpitch2∙cos2θ

=1.2452∙0.235∙25∙1-0.2351.2452∙cos20=63.86

fcage=12∙fr∙1-DballDpitch∙cosθ

=12∙25∙1-0.2351.245∙cos0=10.14

W>2*fsminfOR,fIR,fball,fcage=2*97656/610.14=56.66

TABLE VI

Prediction Accuracies with different parameters(%)

| W | Pt | Min | Max | Mean | Std |
| --- | --- | --- | --- | --- | --- |
| 96 | 0.8 | 100.00 | 100.00 | 100.00 | 0.00 |
| 96 | 0.5 | 100.00 | 100.00 | 100.00 | 0.00 |
| 96 | 0.1 | 99.68 | 100.00 | 99.93 | 0.09 |
| 64 | 0.8 | 99.95 | 100.00 | 99.99 | 0.02 |
| 64 | 0.5 | 99.92 | 100.00 | 99.99 | 0.02 |
| 64 | 0.1 | 99.50 | 99.97 | 99.83 | 0.12 |
| 32 | 0.8 | 99.65 | 99.95 | 99.85 | 0.10 |
| 32 | 0.5 | 99.48 | 99.76 | 99.63 | 0.09 |
| 32 | 0.1 | 97.56 | 98.43 | 97.96 | 0.29 |

To investigate the effect of different parameters on the prediction accuracy, experiments with W=32,64,96 and Pt=0.1,0.5,0.8 are conducted. The results are given in Table.VI. The classification accuracy with W=32 cannot reach 100% because of W<56.66. The max accuracy can reach 100% when W=64 and Pt=0.5 or 0.8. With the increasing of W and Pt, our method will lead to more chance to get 100% accuracy. Furthermore, the results is not sensitive with W and Pt except the situation of 100% accuracy.

## Self-priming Centrifugal Pump(SPCP) Dataset

In this test bed, the acceleration sensor is fixed on a specific pedestal, which rotation speed is 2900r/min. And the sample frequency is 10239Hz. Five categories data are recorded, which are bearing inner race wearing fault(IR), bearing outer race wearing fault(OR), bearing roller wearing fault(Roller), Bi-spectrum counter maps under impeller wearing fault(Impeller) and normal. 10000 data samples are randomly extracted from raw data file with the same probability.

Impeller                      Roller

Normal                     IR

OR

Fig.10.  Signal and images of SPCP data set(W=64)

Fig.10 list the signal, vibration image and power spectrum with W=64. It shows that power spectrum is easier to be discriminated among these categories. So in the comparison with CNN-VibrationImage[7]  and SURF-based PNN[17], our proposed method is the most efficient and reach 100% shown in Table.VII.

TABLE VII

Comparison with Self-priming centrifugal pump dataset (Pt=0.8,%)

| Algorithm | Min | Max | Mean | Std |
| --- | --- | --- | --- | --- |
| W=64 | 100.00 | 100.00 | 100.00 | 0.00 |
| CNN-VibrationImage[7] |  | 99.74 | 99.48 | 0.20 |
| SURF-based PNN[17] |  |  | 98.33 | 1.72 |

## Axial Piston Hydraulic Pump(APHP) Dataset

An accelerograph is installed at the end face of the pump, which rotation speed is 5280r/min. And the sample frequency is 1k Hz. There are three categories signals including normal, valve plate wearing(VPW) and piston shoes and swashplate wearing(PSSW). 4000 data samples are randomly extracted from raw data file with the same probability.

Fig.11 illustrate three kinds signals, vibration images and power spectrum images with W=20. It shows that power spectrum is the easiest to be discriminated among these categories.

Ten times are run with Pt=0.8,W=16,20 and we get the same prediction accuracies with CNN-VibrationImage[7] in Table.VIII.

normal                       PSSW

VPW

Fig. 11.  Signal and imagesof APHP data set(W=20)

TABLE VIII

Comparison with Axial piston hydraulic pump dataset(Pt=0.8, %)

| Algorithm | Min | Max | Mean | Std |
| --- | --- | --- | --- | --- |
| CNN-SFFT(W=16) | 99.87 | 100.00 | 99.99 | 0.03 |
| CNN-SFFT(W=20) | 100.00 | 100.00 | 100.00 | 0.00 |
| CNN-VibrationImage [7] | 100.00 | 100.00 | 100.00 | 0.00 |
| SURF-based PNN[17] |  |  | 98.71 | 1.53 |

## The Probability to Reach 100% Accuracy

In this work, four datasets have been used to verify the efficiency of our proposed method BFD-CNN. By using above experiment’s results, a statistical probability about 100% accuracy within ten running times given in Table.IX is used to investigate the influence of parameters.

TABLE IX

statistical probability about 100% accuracy

| DataSet | W | Pt | P100 |
| --- | --- | --- | --- |
| CWRU | 96 | 0.8 | 1 |
| CWRU | 96 | 0.5 | 0.7 |
| CWRU | 64 | 0.8 | 0.5 |
| MFPT | 96 | 0.8 | 1 |
| MFPT | 96 | 0.5 | 1 |
| MFPT | 96 | 0.1 | 0.3 |
| MFPT | 64 | 0.8 | 0.9 |
| MFPT | 64 | 0.5 | 0.7 |
| SPCP | 64 | 0.8 | 1 |
| APHP | 20 | 0.8 | 1 |
| APHP | 16 | 0.8 | 0.9 |

It can be concluded that the larger W and Pt have big probability to achieve 100% prediction accuracy and W is the domain factor. For example, when CWRU and MFPT dataset are used, W=96 has more chance to get 100% accuracy.

# Conclusion

In this paper, we propose an efficient ubiquitous CNN with tunable input-size W*W for bearing fault diagnosis (BFD-CNN). With the guide of theoretic analysis about the relation between the characteristic vibration frequencies and window size of STFT, it is easier to get the low bound W which decides the input-size of matrix fed to CNN. And with the increasing W, robust results can be obtained with BFD-CNN. But it should be noticed that large W will cause the degradation of the performance because more running time is to be required.

The proposed method is tested with four dataset including two public famous dataset, CWRU and MFPT. After ten times running, all the prediction accuracies of four test data are surprisingly 100%. In these experiments, CNN structure do not need be changed with different dataset. The only thing is just adjust the parameter W and the results are not sensitive with W and Pt, so the BFD-CNN is more general and flexible.

In the future, datasets with more classes of faults are required to further valid the performance of BFD-CNN. And making a step further research to find unknown faults is needed.

References

P. Henriquez, J. B. Alonso, M. A. Ferrer, et al. “Review of automatic fault diagnosis systems using audio and vibration signals,” IEEE Transactions on Systems, Man, and Cybernetics: Systems, vol 44,no 5, pp. 642-652, 2014

A. Rai, S. H. Upadhyay. “A review on signal processing techniques utilized in the fault diagnosis of rolling element bearings,” Tribology International, , vol 96,pp. 289-306. 2016

J. Tian, C. Morillo, M.H. Azarian, M. Pecht, “Motor bearing fault detection using spectral kurtosis-based feature extraction coupled with k-nearest neighbor distance analysis,” IEEE Trans. Ind. Electron, vol 63, no 3, pp. 1793-1803, 2016

Y. Lei, N. Li, L. Guo, et al. “Machinery health prognostics: A systematic review from data acquisition to RUL prediction,” Mechanical Systems and Signal Processing, vol 104, pp. 799-834. , 2018

D. T. Hoang, H. J. Kang, “Rolling element bearing fault diagnosis using convolutional neural network and vibration Image,” Cognitive Systems Research, vol 53, pp. 42-50 , 2019

Z. Gao, C. Cecati, and S. X. Ding, “A survey of fault diagnosis and fault-tolerant techniques-Part I: Fault diagnosis with model-based and signal-based approaches,” IEEE Trans. Ind. Electron., vol. 62, no. 6, pp. 3757–3767, 2015.

L. Wen, X. Li, L. Gao, et al. “A new convolutional neural network-based data-driven fault diagnosis method,” IEEE Transactions on Industrial Electronics, vol 65, no 7, pp. 5990-5998, 2018

A. F. Aimer, A. H. Boudinar, N. Benouzza, et al. “Bearing Fault Diagnosis of a PWM Inverter Fed-Induction Motor Using an Improved Short Time Fourier Transform,” Journal of Electrical Engineering & Technology, pp.1-10. , 2019

D. Verstraete, A. Ferrada, E. L. Droguett, et al. “Deep learning enabled fault diagnosis using time-frequency image analysis of rolling element bearings,” Shock and Vibration , 2017.

X. Zhang, Y. Liang, et al., “A novel bearing fault diagnosis model integrated permutation entropy, ensemble empirical mode decomposition and optimized SVM,” Measurement, vol. 69, pp. 164--179, 2015.

A. Soualhi, K. Medjaher, N. Zerhouni, “Bearing health monitoring based on Hilbert--Huang transform, support vector machine, and regression,” IEEE Transactions on Instrumentation and Measurement, vol. 64, no.1, pp. 52--62, 2015.

U. Jung, and B. Koh, “Wavelet energy-based visualization and classification of high-dimensional signal for bearing fault detection,” Knowledge and Information Systems, vol. 44, no.1, pp. 197--215, 2015.

D. Wang, “K-nearest neighbors based methods for identification of different gear crack levels under different motor speeds and loads: Revisited,” Mechanical Systems and Signal Processing, vol. 70, pp. 201--208, 2016.

J. Seshadrinath, B. Singh, B. Panigrahi, “Vibration analysis based interturn fault diagnosis in induction machines,” IEEE Transactions on Industrial Informatics, vol. 10, no.1, pp. 340--350, 2014.

U. Chong, and others, “Signal model-based fault detection and diagnosis for induction motors using features of vibration signal in two-dimension domain,” Journal of Mechanical Engineering, vol. 57, no.9, pp. 655--666, 2011.

M. Kang and J. M. Kim, “Reliable fault diagnosis of multiple induction motor defects using a 2-D representation of Shannon wavelets,” IEEE Trans. Magn., vol. 50, no. 10, pp. 1–13, 2014.

C. Lu, Y.Wang, M. Ragulskis, and Y. Cheng, “Fault diagnosis for rotating machinery: A method based on image processing,” PloS One, vol. 11,no. 10, 2016, Art. no. e0164111.

R. Zhao, R. Yan, et al., “Deep learning and its applications to machine health monitoring,” Mechanical Systems and Signal Processing, vol. 115, pp. 213--237, 2019.

Y. LeCun, and L. Bottou, “Gradient-based learning applied to document recognition,” Proceedings of the IEEE, vol. 86, no.11, pp. 2278--2324, 1998.

A. Krizhevsky, I. Sutskever, G.E. Hinton, “Imagenet classification with deep convolutional neural networks,” Advances in neural information processing systems, vol 25,no 2, pp. 1097--1105, 2012.

T. Ince, S. Kiranyaz, L. Eren, M. Askar, and M. Gabbouj, “Real-time motor fault detection by 1-D convolutional neural networks,” IEEE Trans. Ind. Electron., vol. 63, no. 11, pp. 7067–7075, 2016.

L. Eren, “Bearing fault detection by one-dimensional convolutional neural networks,” Mathematical Problems in Engineering, 2017.

O. Abdeljaber, O. Avci, S. Kiranyaz, M. Gabbouj, and D. J.Inman, “Real-time vibration-based structural damage detection using one-dimensional convolutional neural networks,” Journal of Sound and Vibration, vol. 388, pp. 154–170, 2017.

X. Guo, L. Chen, and C. Shen, “Hierarchical adaptive deep convolution neural network and its application to bearing fault diagnosis,” Measurement: Journal of the International Measurement Confederation, vol. 93, pp. 490–502, 2016.

F. Jia, Y. Lei, J. Lin, et al. “Deep neural networks: A promising tool for fault characteristic mining and intelligent diagnosis of rotating machinery with massive data,” Mechanical Systems and Signal Processing, vol. 72, pp. 303--315, 2016.

H. Liu, L. Li, J. Ma, ”Rolling bearing fault diagnosis based on STFT-deep learning and sound signals,” Shock and Vibration, 2016.

J. Wang, J. Zhuang, L. Duan, and W. Cheng, “A multi-scale convolution neural network for featureless fault diagnosis,” in Proceedings of the International Symposium on Flexible Automation, pp. 65--70, 2016.

L. Tan, and J. Jiang, “Digital signal processing: fundamentals and applications,” Academic Press, 2013

Z. Zhang, and M. Sabuncu, ”Generalized cross entropy loss for training deep neural networks with noisy labels,” Advances in Neural Information Processing Systems, pp. 8778--8788, 2018.

D. P. Kingma, and J. Ba, ”Adam: A method for stochastic optimization,” in International Conference on Learning Representations, 2015.

K. A. Loparo, Loparo, K. A., “Bearing Data Center,” Case Western Reserve University, http://csegroups.case.edu/bearingdatacenter/ pages/welcome-case-western-reserve-university-bearing-datacenter- website, 2013.

E. Bechhoefer, “A Quick Introduction to Bearing Envelope Analysis,” MFPT Data, http://www.mfpt.org/FaultData/Fault- Data.htm.Set, 2016.

D. Lee, et al., “Convolutional neural net and bearing fault analysis,” in Proceedings of the International Conference on Data Mining (DMIN), 2016.

H. Jiang, X. Li, H. Shao, “Intelligent fault diagnosis of rolling bearings using an improved deep recurrent neural network,” Measurement Science and Technology, vol. 29, no.6, pp. 065107, 2018.

Y. Lei, F. Jia, J. Lin, S. Xing, and S. X. Ding, “An intelligent fault diagnosis method using unsupervised feature learning towards mechanical big data,” IEEE Trans. Ind. Electron., vol. 63, no. 5, pp. 3137–3147, 2016

H. Shao, H. Jiang, X. Zhang, andM. Niu, “Rolling bearing fault diagnosis using an optimization deep belief network,” Meas. Sci. Technol., vol. 26, no. 11, Sep. 2015, Art. no. 115002.

M. Gan and C. Wang, “Construction of hierarchical diagnosis network based on deep learning and its application in the fault pattern recognition of rolling element bearings,” Mech. Syst. Signal Process., vol. 72, pp. 92–104, May 2016.

W. A. Smith, and R. B. Randall, “Rolling element bearing diagnostics using the Case Western Reserve University data: A benchmark study,” Mechanical Systems and Signal Processing, vol. 64, pp. 100--131, 2015.